Procedure for Firmware Upgrade

In the following text unit can be RX1+1, TX1+1, or IBUC.

Ethernet Upgrade:

This procedure requires a PC connected to a LAN. The PC should be on the same LAN or directly connected to the unit being upgraded.

ZIP FILE:

Begin the upgrade by navigating to the directory you extracted the files to inside Windows Explorer and double clicking UDPDownload.exe with the left mouse button.

You should now see a screen similar (file names may be different) to the following screen:

If you are connected to the same subnet as the unit, the utility will automatically detect any unit connected.

Note: If you are not located on the same LAN as the unit, then select the Ping->Ping Specific IP from the menu. Type in the IP address of the unit you wish to upgrade and press the OK button. If the unit is reachable from your location, it will appear in the window above.

Once the unit you wish to upgrade is in the window, with the mouse left button single click on the entry that matches the unit you wish to upgrade. The entry should be highlighted in blue.

Note: If you are upgrading a TX 1+1 system select the offline unit to upgrade first to minimize switching. Once the offline unit is upgraded and has rebooted, then upgrade the online unit.

- Press the “…” button located to the right of  “Ram Loader” window.

- Navigate to the directory where the UDPDownload.exe is located and select the file DL-SMSC-D_2005_02_20.bin.

- Press the “…” button located to the right of the “User Program:” window.

- Navigate to the directory where the firmware you wish to download is located and select it.

- Verify “Query for user block size” is checked.

- Verify “Force RAM loader Download” is unchecked.

- Verify “Force Short Packets” is unchecked.

- Verify “Specify RAM loader” is selected.

Click on the Download menu option to start the upgrade. Once the upgrade is complete you will see a message in the window below the units telling you the unit is rebooting.

Repeat the procedure for any additional units requiring upgrade.

Serial Upgrade:

This procedure requires a direct connection from a PC COM port to the unit being upgraded. Verify the unit password is numeric in the range 11111-65535. You will not be able to access the unit after the upgrade otherwise.

ZIP FILE:

Begin the upgrade by navigating to the directory you extracted the files to inside Windows Explorer and double clicking serdownload.exe with the left mouse button.

You should now see a screen similar (file names may be different) to the following screen:

Note: Be sure to check the box labeled “Use Short Packets”.

- Select the “Serial Port” you wish to use.

- Select 9600 baud in the “Baud Rate” window.

- Press the “Connect” button.

- Press the “Terminal” button. When the terminal window opens type CPE=<Password> followed by the enter key on your PC keyboard. <Password> represents the password of your unit. Next type DLD followed by the enter key on your PC keyboard.

- Change the “Baud Rate window to 115200.

- Press the “Connect” button.

- Press the “Search” button. The unit information should appear in the “Board Status” window.

- Press the “…” button to the right of the “RAM loader:” box. Navigate to the directory where the serdowmload.exe is located and select the file serloader.bin.

- Press the “…” button to the right of the “User Program:” box. Navigate to the appropriate software folder for the unit you are upgrading and select the bin file located there.

- Press the “Download” button and the download will commence. At the end of the download procedure the unit will automatically reset and start the new Firmware.

Note: If you are upgrading a TX 1+1 system upgrade the offline unit to minimize switching. Once the offline unit is upgraded and has rebooted, then upgrade the online unit.

Repeat the procedure for any additional units requiring upgrade.